PROJECT ID: CAFS.21.88

YEAR: 2

PROJECT TITLE: Quantifying silvicultural treatment effect on lumber quantity and quality in loblolly pine

INVESTIGATOR(S): Joe Dahlen (UGA), Cristian Montes (UGA), Bronson Bullock (UGA), Corey Green (VT), Harold Burkhardt (VT), Tom Eberhardt (USFS)

PROJECT DESCRIPTION: Even though over 18 billion board feet of southern pine lumber are produced each year from the U.S. south, information on silvicultural treatment impacts on lumber quantity and lumber is very limited. This is because forest through mill studies are difficult to conduct and thus rarely done. This study will investigate the impact of silvicultural treatment on the lumber quantity and quality from loblolly pine from the Forest Modeling Research Cooperative’s Regionwide Intensively Managed (IMP) study. The study has 3 treatments, a control treatment with no thinning or pruning, a light thinning treatment where 1/3 of the trees have been removed, and a heavy thinning treatment where 2/3 of the trees were removed and the first log was pruned.

HYPOTHESES or OBJECTIVES: The major research question is will the heavy thinning treatment plus pruning the first log result in more value than the control or light thinning treatments?

METHODS: Trees will be felled from five sites in the Virginia Piedmont. From each site, 7 trees per treatment for a total of 21 trees per site and 105 trees overall will be felled, cut into 17 feet logs, and processed through a lumber mill into dimension lumber. The lumber quantity and quality will be measured via nondestructive testing and destructive testing to investigate the impact of silvicultural treatment on product quality.

MAJOR FINDINGS: Lumber has been sawn from the trees harvested. A total of 1,100 pieces were obtained. We are currently working on the extensive laboratory work related to this project.

DELIVERABLES: The volume, grade and mechanical properties of lumber will be measured to determine silvicultural treatment effects.

MEMBER COMPANY BENEFITS: Information on lumber quality based on silvicultural treatment. Relationships between nondestructive tests and destructive tests.